TUTORIAL

Simultaneous Localization and Mapping: Part I

BY HUGH DURRANT-WHYTE AND TIM BAILEY

The simultaneous localization and mapping (SLAM)

problem asks if it is possible for a mobile robot to be placed at an unknown location in an unknown envi ronment and for the robot to incrementally build a consistent map of this environment while simultaneously determining its location within this map. A solution to the SLAM problem has been seen as a “holy grail” for the mobile robotics com munity as it would provide the means to make a robot truly autonomous.

The “solution” of the SLAM problem has been one of the notable successes of the robotics community over the past decade. SLAM has been formulated and solved as a theoretical problem in a number of different forms. SLAM has also been implemented in a number of different domains from indoor robots to outdoor, underwater, and airborne systems. At a theoretical and conceptual level, SLAM can now be consid ered a solved problem. However, substantial issues remain in practically realizing more general SLAM solutions and notably in building and using perceptually rich maps as part of a SLAM algorithm.

This two-part tutorial and survey of SLAM aims to provide a broad introduction to this rapidly growing field. Part I (this article) begins by providing a brief history of early develop ments in SLAM. The formulation section introduces the struc ture the SLAM problem in now standard Bayesian form, and explains the evolution of the SLAM process. The solution sec tion describes the two key computational solutions to the SLAM problem through the use of the extended Kalman filter (EKF-SLAM) and through the use of Rao-Blackwellized par ticle filters (FastSLAM). Other recent solutions to the SLAM problem are discussed in Part II of this tutorial. The application section describes a number of important real-world implemen tations of SLAM and also highlights implementations where the sensor data and software are freely down-loadable for other researchers to study. Part II of this tutorial describes major issues in computation, convergence, and data association in SLAM. These are subjects that have been the main focus of the SLAM research community over the past five years.

History of the SLAM Problem

The genesis of the probabilistic SLAM problem occurred at the 1986 IEEE Robotics and Automation Conference held in San Francisco, California. This was a time when probabilistic

methods were only just beginning to be introduced into both robotics and artificial intelligence (AI). A number of researchers had been looking at applying estimation-theoretic methods to mapping and localization problems; these includ ed Peter Cheeseman, Jim Crowley, and Hugh Durrant Whyte. Over the course of the conference, many paper table cloths and napkins were filled with long discussions about consistent mapping. Along the way, Raja Chatila, Oliver Faugeras, Randal Smith, and others also made useful contri butions to the conversation.

The result of this conversation was a recognition that consistent probabilistic mapping was a fundamental problem in robotics with major conceptual and computational issues that needed to be addressed. Over the next few years a number of key papers were produced. Work by Smith and Cheesman [39] and Durrant-Whyte [17] established a statis tical basis for describing relationships between landmarks and manipulating geometric uncertainty. A key element of this work was to show that there must be a high degree of cor relation between estimates of the location of different land marks in a map and that, indeed, these correlations would grow with successive observations.

At the same time Ayache and Faugeras [1] were under taking early work in visual navigation, Crowley [9] and Chatila and Laumond [6] were working in sonar-based navi gation of mobile robots using Kalman filter-type algorithms. These two strands of research had much in common and resulted soon after in the landmark paper by Smith et al. [40]. This paper showed that as a mobile robot moves through an unknown environment taking relative observa tions of landmarks, the estimates of these landmarks are all necessarily correlated with each other because of the com mon error in estimated vehicle location [27]. The implica tion of this was profound: A consistent full solution to the combined localization and mapping problem would require a joint state composed of the vehicle pose and every land mark position, to be updated following each landmark observation. In turn, this would require the estimator to employ a huge state vector (on the order of the number of landmarks maintained in the map) with computation scaling as the square of the number of landmarks.

Crucially, this work did not look at the convergence properties of the map or its steady-state behavior. Indeed, it

JUNE 2006 IEEE Robotics & Automation Magazine 99

1070-9932/06/$20.00©2006 IEEE

Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

was widely assumed at the time that the estimated map errors would not converge and would instead exhibit a ran dom-walk behavior with unbounded error growth. Thus, given the computational complexity of the mapping prob lem and without knowledge of the convergence behavior of the map, researchers instead focused on a series of approxi mations to the consistent mapping problem, which assumed or even forced the correlations between landmarks to be minimized or eliminated, so reducing the full filter to a series of decoupled landmark to vehicle filters ([28] and [38] for example). Also for these reasons, theoretical work on the combined localization and mapping problem came to a tem porary halt, with work often focused on either mapping or localization as separate problems.

The conceptual breakthrough came with the realization that the combined mapping and localization problem, once formulated as a single estimation problem, was actually con vergent. Most importantly, it was recognized that the corre lations between landmarks, which most researchers had tried to minimize, were actually the critical part of the problem and that, on the contrary, the more these correlations grew, the better the solution. The structure of the SLAM problem, the convergence result and the coining of the acronym SLAM was first presented in a mobile robotics survey paper presented at the 1995 International Symposium on Robotics Research [16]. The essential theory on convergence and many of the initial results were developed by Csorba [10],

[11]. Several groups already working on mapping and local ization, notably at the Massachusetts Institute of Technology [29], Zaragoza [4], [5], the ACFR at Sydney [20], [45], and others [7], [13], began working in earnest on SLAM—also called concurrent mapping and localization (CML) at this time— in indoor, outdoor, and subsea environments.

At this time, work focused on improving computational efficiency and addressing issues in data association, or loop closure. The 1999 International Symposium on Robotics Research (ISRR’99) [23] was an important meeting point where the first SLAM session was held and where a degree of convergence between the Kalman-filter-based SLAM methods and the probabilistic localisation and mapping methods introduced by Thrun [42] was achieved. The 2000 IEEE International Conference on Robotics and Automation (ICRA) Workshop on SLAM attracted 15 researchers and focused on issues such as algorithmic com plexity, data association, and implementation challenges. The following SLAM workshop at the 2002 ICRA attract ed 150 researchers with a broad range of interests and applications. The 2002 SLAM summer school hosted by Henrik Christiansen at KTH in Stockholm attracted all the key researchers together with some 50 Ph.D. students from around the world and was a tremendous success in building the field. Interest in SLAM has grown exponentially in recent years, and workshops continue to be held at both ICRA and IROS. The SLAM summer school ran in 2004 in Toulouse and will run at Oxford,

England, in 2006.

xk

xk−1

zk,j

uk

mj

xk+1

uk+1

xk+2

uk+2

Formulation and Structure of the SLAM Problem

SLAM is a process by which a mobile robot can build a map of an environ ment and at the same time use this map to deduce its location. In SLAM, both the trajectory of the platform and the location of all landmarks are estimated online without the need for any a priori knowledge of location.

Preliminaries

zk−1,i

mi

Robot Landmark

Estimated

True

Consider a mobile robot moving through an environment taking relative observa tions of a number of unknown landmarks using a sensor located on the robot as shown in Figure 1. At a time instant k, the following quantities are defined:

◆ xk : the state vector describing the location and orientation of the vehicle

Figure 1. The essential SLAM problem. A simultaneous estimate of both robot and landmark locations is required. The true locations are never known or measured directly. Observations are made between true robot and landmark locations.

◆ uk : the control vector, applied at time k − 1 to drive the vehicle to a state xk at time k

100 IEEE Robotics & Automation Magazine JUNE 2006 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

◆ mi: a vector describing the location of the ith landmark whose true location is assumed time invariant ◆ zik : an observation taken from the vehicle of the location of the ith landmark at time k. When there are multiple landmark observations at any one time or

The SLAM algorithm is now implemented in a standard two-step recursive (sequential) prediction (time-update) cor rection (measurement-update) form:

Time-update

when the specific landmark is not relevant to the dis cussion, the observation will be written simply as zk .

P(xk, m|Z0:k−1, U0:k, x0) =

P(xk|xk−1, uk)

In addition, the following sets are also defined:

◆ X0:k = {x0, x1,··· , xk}={X0:k−1, xk}: the history of vehicle locations

◆ U0:k = {u1, u2,··· , uk}={U0:k−1, uk}: the history of control inputs

◆ m = {m1, m2,··· , mn} the set of all landmarks ◆ Z0:k = {z1, z2,··· , zk}={Z0:k−1, zk}: the set of all landmark observations.

Probabilistic SLAM

In probabilistic form, the simultaneous localization and map building (SLAM) problem requires that the probability dis tribution

P(xk, m|Z0:k, U0:k, x0) (1)

be computed for all times k. This probability distribution describes the joint posterior density of the landmark locations and vehicle state (at time k) given the recorded observations and control inputs up to and including time k together with the initial state of the vehicle. In general, a recursive solution to the SLAM problem is desirable. Starting with an estimate for the distribution P(xk−1, m|Z0:k−1, U0:k−1) at time k − 1, the joint posterior, following a control uk and obser vation zk , is computed using Bayes theorem. This computa tion requires that a state transition model and an observation model are defined describing the effect of the control input and observation respectively.

The observation model describes the probability of making an observation zk when the vehicle location and landmark loca tions are known and is generally described in the form

P(zk|xk, m). (2)

It is reasonable to assume that once the vehicle location and map are defined, observations are conditionally independent given the map and the current vehicle state.

The motion model for the vehicle can be described in terms of a probability distribution on state transitions in the form

P(xk|xk−1, uk). (3)

That is, the state transition is assumed to be a Markov process in which the next state xk depends only on the immediately preceding state xk−1 and the applied control uk and is inde pendent of both the observations and the map.

× P(xk−1, m|Z0:k−1, U0:k−1, x0)dxk−1 (4) Measurement Update

P(xk, m|Z0:k, U0:k, x0)

= P(zk|xk, m)P(xk, m|Z0:k−1, U0:k, x0)

P(zk|Z0:k−1, U0:k) (5)

Equations (4) and (5) provide a recursive procedure for calcu lating the joint posterior P(xk, m|Z0:k, U0:k, x0) for the robot state xk and map m at a time k based on all observa tions Z0:k and all control inputs U0:k up to and including time k. The recursion is a function of a vehicle model P(xk|xk−1, uk) and an observation model P(zk|xk, m).

It is worth noting that the map building problem may be formulated as computing the conditional density P(m|X0:k, Z0:k, U0:k). This assumes that the location of the vehicle xk is known (or at least deterministic) at all times, sub ject to knowledge of initial location. A map m is then con structed by fusing observations from different locations. Conversely, the localization problem may be formulated as computing the probability distribution P(xk|Z0:k, U0:k, m). This assumes that the landmark locations are known with cer tainty, and the objective is to compute an estimate of vehicle location with respect to these landmarks.

Structure of Probabilistic SLAM

To simplify the discussion in this section, we will drop the conditioning on historical variables in (1) and write the required joint posterior on map and vehicle location as P(xk, m|zk) and even P(xk, m) as the context permits.

The observation model P(zk|xk, m) makes explicit the dependence of observations on both the vehicle and landmark locations. It follows that the joint posterior cannot be parti tioned in the obvious manner

P(xk, m|zk)  = P(xk|zk)P(m|zk),

and indeed it is well known from the early papers on consistent mapping [17], [39] that a partition such as this leads to inconsistent estimates. However, the SLAM problem has more structure than is immediately obvious from these equations.

Referring again to Figure 1, it can be seen that much of the error between estimated and true landmark locations is common between landmarks and is in fact due to a single

JUNE 2006 IEEE Robotics & Automation Magazine 101 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

source; errors in knowledge of where the robot is when land mark observations are made. In turn, this implies that the errors in landmark location estimates are highly correlated. Practically, this means that the relative location between any two landmarks, mi − mj, may be known with high accuracy, even when the absolute location of a landmark mi is quite uncertain. In probabilistic form, this means that the joint probability density for the pair of landmarks P(mi, mj) is highly peaked even when the marginal densities P(mi) may be quite dispersed.

The most important insight in SLAM was to realize that the correlations between landmark estimates increase monoton ically as more and more observations are made. (These results have only been proved for the linear Gaussian case [14]. For mal proof for the more general probabilistic case remains an open problem.) Practically, this means that knowledge of the relative location of landmarks always improves and never diverges, regardless of robot motion. In probabilistic terms, this means that the joint probability density on all landmarks P(m) becomes monotonically more peaked as more observa tions are made.

This convergence occurs because the observations made by the robot can be considered as “nearly independent” mea surements of the relative location between landmarks. Refer ring again to Figure 1, consider the robot at location xk observing the two landmarks mi and mj. The relative loca tion of observed landmarks is clearly independent of the coor dinate frame of the vehicle, and successive observations from this fixed location would yield further independent measure ments of the relative relationship between landmarks. Now, as

Estimated Robot

Estimated Landmark

Correlations

Figure 2. Spring network analogy. The landmarks are connected by springs describing correlations between landmarks. As the vehicle moves back and forth through the environment, spring stiffness or correlations increase (red links become thicker). As landmarks are observed and estimated locations are corrected, these changes are propagated through the spring network. Note, the robot itself is correlated to the map.

the robot moves to location xk+1, it again observes landmark mj this allows the estimated location of the robot and land mark to be updated relative to the previous location xk . In turn, this propagates back to update landmark mi—even though this landmark is not seen from the new location. This occurs because the two landmarks are highly correlated (their relative location is well known) from previous measurements. Further, the fact that the same measurement data is used to update these two landmarks makes them more correlated. The term nearly independent measurement is appropriate because the observation errors will be correlated through successive vehi cle motions. Also note that in Figure 1 at location xk+1, the robot observes two new landmarks relative to mj. These new landmarks are thus immediately linked or correlated to the rest of the map. Later updates to these landmarks will also update landmark mj and through this landmark mi and so on. That is, all landmarks end up forming a network linked by relative location or correlations whose precision or value increases whenever an observation is made.

This process can be visualized (Figure 2) as a network of springs connecting all landmarks together or as a rubber sheet in which all landmarks are embedded. An observation in a neighborhood acts like a displacement to a spring system or rubber sheet such that its effect is great in the neighborhood and, dependent on local stiffness (correlation) properties, diminishes with distance to other landmarks. As the robot moves through this environment and takes observations of the landmarks, the springs become increasingly (and monoto nically) stiffer. In the limit, a rigid map of landmarks or an accurate relative map of the environment is obtained. As the map is built, the location accuracy of the robot measured rel ative to the map is bounded only by the quality of the map and relative measurement sensor. In the theoretical limit, robot relative location accuracy becomes equal to the local ization accuracy achievable with a given map.

Solutions to the SLAM Problem

Solutions to the probabilistic SLAM problem involve find ing an appropriate representation for both the observation model (2) and motion model (3) that allows efficient and consistent computation of the prior and posterior distribu tions in (4) and (5). By far, the most common representa tion is in the form of a state-space model with additive Gaussian noise, leading to the use of the extended Kalman filter (EKF) to solve the SLAM problem. One important alternative representation is to describe the vehicle motion model in (3) as a set of samples of a more general non Gaussian probability distribution. This leads to the use of the Rao-Blackwellized particle filter, or FastSLAM algo rithm, to solve the SLAM problem. While EKF-SLAM and FastSLAM are the two most important solution methods, newer alternatives, which offer much potential, have been proposed, including the use of the information-state form [43]. These are discussed further in Part II of this tutorial.

102 IEEE Robotics & Automation Magazine JUNE 2006 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

EKF-SLAM

The basis for the EKF-SLAM method is to describe the vehi cle motion in the form

P(xk|xk−1, uk) ⇐⇒ xk = f(xk−1, uk) + wk, (6)

where f(·) models vehicle kinematics and where wk are addi tive, zero mean uncorrelated Gaussian motion disturbances with covariance Qk . The observation model is described in the form

P(zk|xk, m) ⇐⇒ zk = h(xk, m) + vk, (7)

where h(·) describes the geometry of the observation and where vk are additive, zero mean uncorrelated Gaussian observation errors with covariance Rk .

With these definitions, the standard EKF method [14], [31] can be applied to compute the mean

and where ∇h is the Jacobian of h evaluated at xˆ k|k−1 and mˆ k−1.

This EKF-SLAM solution is very well known and inherits many of the same benefits and problems as the standard EKF solutions to navigation or tracking problems. Four of the key issues are briefly discussed next.

Convergence

In the EKF-SLAM problem, convergence of the map is manifest in the monotonic convergence of the determinant of the map covariance matrix Pmm,k and all landmark pair submatrices, toward zero. The individual landmark variances converge toward a lower bound determined by initial uncer tainties in robot position and observations. The typical con vergence behavior of landmark location variances is shown in Figure 3 (from [14]).

Computational Effort

,

and covariance

xˆ k|k mˆ k

= E

xk

m |Z0:k

The observation update step requires that all landmarks and the joint covariance matrix be updated every time an observa tion is made. Naively, this means computation grows quadrat ically with the number of landmarks. There has been a great deal of work undertaken in developing efficient variants of

Pk|k =

Pxx Pxm PTxm Pmm

the EKF-SLAM solution and real-time implementations with

many thousands of landmarks have been demonstrated [21],

= E

xk − ˆxk m − ˆmk

k|k

xk − ˆxk m − ˆmk

T

| Z0:k

[29]. Efficient variants of the EKF-SLAM algorithm are dis

cussed in Part II of this tutorial.

of the joint posterior distribution P(xk, m|Z0:k, U0:k, x0) 2.5

from:

Time-update

m)

2

xˆ k|k−1 = f(xˆ k−1|k−1, uk) (8)

X (

Standard Deviation in

Pxx,k|k−1 = ∇f Pxx,k−1|k−1∇fT + Qk, (9)

1.5

where ∇f is the Jacobian of f evaluated at the estimate 1

xˆ k−1|k−1 . There is generally no need to perform a time update for stationary landmarks. (A time-update, however, is 0.5

necessary for landmarks that may move [44].)

Observation-update

xˆ k|k mˆ k

= [ xˆ k|k−1mˆ k−1 ] + Wk[zk − h(xˆ k|k−1, mˆ k−1)] (10)

0

40 50 60 70 80 90 100 110 Time (s)

Pk|k = Pk|k−1 − WkSkWTk , (11)

where

Sk = ∇hPk|k−1∇hT + Rk

Wk = Pk|k−1∇hTS−1

k

Figure 3. The convergence in landmark uncertainty. The  plot shows a time history of standard deviations of a set of landmark locations. A landmark is initially observed with uncertainty inherited from the robot location and observation. Over time, the standard deviations reduce monotonically to a lower bound. New landmarks are acquired during motion (from [14]).

JUNE 2006 IEEE Robotics & Automation Magazine 103 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

Data Association

The standard formulation of the EKF-SLAM solution is espe cially fragile to incorrect association of observations to land marks [35]. The loop-closure problem, when a robot returns to reobserve landmarks after a large traverse, is especially diffi cult. The association problem is compounded in environ ments where landmarks are not simple points and indeed look different from different viewpoints. Current work in this area will be described in Part II of this tutorial.

Nonlinearity

EKF-SLAM employs linearized models of nonlinear motion and observation models and so inherits many caveats. Non linearity can be a significant problem in EKF-SLAM and leads to inevitable, and sometimes dramatic, inconsistency in solutions [24]. Convergence and consistency can only be guaranteed in the linear case.

Rao-Blackwellized Filter

The FastSLAM algorithm, introduced by Montemerlo et al. [32], marked a fundamental conceptual shift in the design of recursive probabilistic SLAM. Previous efforts focused on improving the performance of EKF-SLAM, while retaining its essential linear Gaussian assumptions. FastSLAM, with its basis in recursive Monte Carlo sampling, or particle filtering, was the first to directly represent the nonlinear process model and non-Gaussian pose distribution. (FastSLAM still linearizes the observation model, but this is typically a rea sonable approximation for range-bearing measurements when the vehicle pose is known.) This approach was influ enced by earlier probabilistic mapping experiments of Mur phy [34] and Thrun [41].

space by applying Rao-Blackwellization (R-B), whereby a joint state is partitioned according to the product rule P(x1, x2) = P(x2|x1)P(x1) and, if P(x2|x1) can be repre sented analytically, only P(x1) need be sampled x(i)1 ∼ P(x1). The joint distribution, therefore, is represented by the set {x(i)1 , P(x2|x(i)1 }Ni and statistics such as the marginal

P(x2) ≈ 1N NiP x2|x(i)1

can be obtained with greater accuracy than is possible by sam pling over the joint space.

The joint SLAM state may be factored into a vehicle com ponent and a conditional map component:

P(X0:k, m|Z0:k, U0:k, x0)

= P(m|X0:k, Z0:k)P(X0:k|Z0:k, U0:k, x0). (12)

Here the probability distribution is on the trajectory X0:k rather than the single pose xk because, when conditioned on the trajectory, the map landmarks become independent (see Figure 4). This is a key property of FastSLAM and the reason for its speed; the map is represented as a set of independent Gaussians, with linear complexity, rather than a joint map covariance with quadratic complexity.

The essential structure of FastSLAM, then, is a Rao Blackwellized state, where the trajectory is represented by weighted samples and the map is computed analytically. Thus, the joint distribution, at time k, is represented by the set {w(i)k , X(i)0:k, P(m|X(i)0:k, Z0:k)}Ni , where the map accompa nying each particle is composed of independent Gaussian

distributions

The high dimensional state-space of the SLAM problem

makes direct application of particle filters computationally P

infeasible. However, it is possible to reduce the sample

m|X(i)0:k, Z0:k =  M j

P

mj|X(i)0:k, Z0:k .

uk uk+1 uk+2

xk−1 xk xk+1 xk+2

zk−1,i zk,i zk+1,i

mi

Figure 4. A graphical model of the SLAM algorithm. If the his tory of pose states are known exactly then, since the observa tions are conditionally independent, the map states are also independent. For FastSLAM, each particle defines a different vehicle trajectory hypothesis.

Recursive estimation is performed by particle filtering for the pose states and the EKF for the map states.

Updating the map, for a given trajectory particle X(i)0:k , is triv ial. Each observed landmark is processed individually as an EKF measurement update from a known pose (see Figure 5). Unob served landmarks are unchanged. Propagating the pose particles, on the other hand, is more complex, as we discuss below.

We forego giving the background on particle filters, except to say that the theory is derived from a recursive form of sam pling known as sequential important sampling (SIS) [15], which actually samples from a joint state history but “telescopes’’ the joint into a recursion via the product rule.

P(x0, x1,..., xT|Z0:T)

= P(x0|Z0:T)P(x1|x0, Z0:T)... P(xT|X0:T−1, Z0:T).

At each time-step k, particles are drawn from a proposal dis tribution π(xk|X0:k−1, Z0:k), which approximates the true

104 IEEE Robotics & Automation Magazine JUNE 2006 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

distribution P(xk|X0:k−1, Z0:T), and the samples are given importance weights to compensate for any discrepancy. The approximation error grows with time (and inherent joint state space), increasing the variation in sample weights, degrading statistical accuracy. A resampling step reinstates uniform weighting, but causes loss of historical particle information. This leads to a crucial property: SIS with resampling can produce reasonable statistics only for systems that “exponentially forget’’ their past [8] (i.e., systems whose process noise cause the state at time k to become increasingly independent of preceding states).

The general form of a R-B particle filter for SLAM is as follows. We assume that, at time k − 1, the joint state is rep resented by {w(i)k−1, X(i)0:k−1, P(m|X(i)0:k−1, Z0:k−1)}Ni .

1) For each particle, compute a proposal distribution, con ditioned on the specific particle history, and draw a sample from it

x(i)k ∼ π xk|X(i)0:k−1, Z0:k, uk . (13)

This new sample is (implicitly) joined to the particle history X(i)0:k = {X(i)0:k−1, x(i)k }.

2) Weight samples according to the importance function

Figure 5. A single realization of robot trajectory in the Fast SLAM algorithm. The ellipsoids show the proposal distribution for each update stage, from which a robot pose is sampled,

w(i )

k = w(i ) k−1

P

zk|X(i )

0:k, Z0:k−1

x(i )

P

k |x(ik−1, uk

x(i )

. (14)

and, assuming this pose is perfect, the observed landmarks are updated. Thus, the map for a single particle is governed by the accuracy of the trajectory. Many such trajectories provide a probabilistic model of robot location.

π

k |X(i )

0:k−1, Z0:k, uk

The numerator terms of this equation are the observa tion model and the motion model, respectively. The former differs from (2) because R-B requires dependen

For FastSLAM 1.0, the proposal distribution is the motion model

cy on the map be marginalized away. P(zk|X0:k, Z0:k−1)

x(i )

k ∼ P

xk|x(i )

k−1, uk

. (16)

=

P(zk|xk, m)P(m|X0:k−1, Z0:k−1)dm (15)

Therefore, from (14), the samples are weighted according to the marginalized observation model.

3) If necessary, perform resampling. (When best to insti gate resampling is an open problem. Some implementa

w(i )

k = w(i )

k−1P

zk|X(i )

0:k, Z0:k−1

(17)

tions resample every time-step, others after a fixed number of time-steps, and others once the weight vari

For FastSLAM 2.0, the proposal distribution includes the cur rent observation

ance exceeds a threshold.) Resampling is accomplished by selecting particles, with replacement, from the set {X(i )

0:k}Ni , including their associated maps, with proba bility of selection proportional to w(i )

x(i )

k ∼ P

xk|X(i )

0:k−1, Z0:k, uk

, (18)

are given uniform weight, w(i )

k . Selected particles

where

k = 1/N .

P

4) For each particle, perform an EKF update on the

xk|X(i )

0:k−1, Z0:k, uk

observed landmarks as a simple mapping operation with known vehicle pose.

The two versions of FastSLAM in the literature, Fast

= 1CP zk|xk, X(i )

0:k−1, Z0:k−1

P

xk|x(i )

k−1, uk

,

SLAM 1.0 [32] and FastSLAM 2.0 [33], differ only in terms of the form of their proposal distribution (Step 1) and, conse quently, in their importance weight (Step 2). FastSLAM 2.0 is by far the more efficient solution.

and C is a normalizing constant. The importance weight according to (14) is w(i )

k = w(i )

k−1C. The advantage of Fast SLAM 2.0 is that its proposal distribution is locally optimal [15]. That is, for each particle, it gives the smallest possible

JUNE 2006 IEEE Robotics & Automation Magazine 105 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

variance in importance weight w(i )

Implementation of SLAM

available information, X(i )

k conditioned upon the

0:k−1, Z0:k and U0:k .

Statistically, FastSLAM (1.0 and 2.0) suffers degeneration due to its inability to forget the past. Marginalizing the map in (15) introduces dependence on the pose and measurement history, and so, when resampling depletes this history, statisti cal accuracy is lost [2]. Nevertheless, empirical results of Fast SLAM 2.0 in real outdoor environments [33] show that the algorithm is capable of generating an accurate map in practice.

Practical realizations of probabilistic SLAM have become increasingly impressive in recent years, covering larger areas in more challenging environments. Here we discuss two representative implementations and mention other notable applications.

The “explore and return’’ experiment by Newman et al. [37] was a moderate-scale indoor implementation that vali dated the nondivergence properties of EKF-SLAM by returning to a precisely marked starting

point. The experiment is remarkable

MIT-CML P. Newman May 2001

MODE:Real Time CML F[2]

because its return trip was fully autonomous. The robot was manually driven during the exploration phase, although without visual contact by the

F[3]F[1]

F[5] F[1]

B21R

F[4] F[13] F[4]

operator, who relied solely on a real time rendering of the robot’s map (see Figure 6). For the return trip, the robot plans a path and returns without human intervention.

Guivant and Nebot [21] pioneered the application of SLAM in very large outdoor environments (see Figure 7). They addressed computational issues of real-time operation, while also dealing with high-speed vehicle motion, non flat terrain, and dynamic clutter. Their results are particularly interesting because they are accompanied by

B21R : X = −1.196,y= 2.091,z= 0.000,h=53.228,Pxx= 0.148,Pyy= 0.071,Sic=y,Sor Figure 6. A real-time SLAM visualization by Newman et al. [37].

accurate RTK-GPS ground truth, showing the practical veracity of the algorithm, which involved closing sev eral large loops. The logged data from

250 Map and Path 200

150

Latitude (m)

100

50

0

−50

their Victoria Park trials is available online and has become a popular benchmark within the SLAM research community.

SLAM applications now exist in a wide variety of domains. They include indoor [3], [4], [7], [12], out door [19], [21], aerial [25], and subsea [18], [36], [45]. There are different sensing modalities such as bearing only [13] and range only [30].

We also make honorable mention of consistent pose estimation (CPE) [22], [26], which is an entirely different SLAM paradigm based on topological map ping and data alignment, due to its exemplary results in large indoor environments.

Various researchers in the SLAM community have written software demonstrating SLAM, implemented in MATLAB, C++, and Java and

−100

−150 −100 −50 0 50 100 150 Longitude (m)

available online (see Table 1). Collections of logged data are listed in Table 2. These datasets are from real sensors in real environments and are a valuable resource to assess and benchmark the var

Figure 7. Large-scale outdoor SLAM by Guivant and Nebot [21]. ious SLAM algorithms. 106 IEEE Robotics & Automation Magazine JUNE 2006 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

Table 1. Open-source SLAM software.

Author Description Link

Kai Arras The CAS Robot Navigation Toolbox, a MATLAB http://www.cas.kth.se/toolbox/index.html simulation toolbox for robot localization and mapping

Tim Bailey MATLAB simulators for EKF-SLAM, UKF-SLAM, and http://www.acfr.usyd.edu.au/homepages/  FastSLAM 1.0 and 2.0. academic/tbailey/software/index.html Mark Paskin Java library with several SLAM variants, including Kalman http://www.stanford.edu/~paskin/slam/ filter, information filter, and thin junction tree forms.

Includes a MATLAB interface.

Andrew Davison Scene, a C++ library for map-building and localization. http://www.doc.ic.ac.uk/~ajd/Scene/ index.html Facilitates real-time single camera SLAM.

José Neira MATLAB EKF-SLAM simulator that demonstrates joint http://webdiis.unizar.es/~neira/  compatibility branch-and-bound data association. software/slam/slamsim.htm

Dirk Hähnel C language grid-based version of FastSLAM. http://www.informatik.uni-freiburg.de/  ~haehnel/old/download.html

Various MATLAB code from the 2002 SLAM summer school. http://www.cas.kth.se/slam/toc.html

Table 2. Online datasets.

Author Description Link

Eduardo Nebot Numerous large-scale outdoor datasets, notably http://www.acfr.usyd.edu.au/homepages/  the popular Victoria Park data. academic/enebot/dataset.htm

Chieh-Chih Wang Three large-scale outdoor datasets collected by the http://www.cs.cmu.edu/~bobwang/ datasets.html Navlab11 testbed.

Radish (The Robotics Many and varied indoor datasets, including large-area http://radish.sourceforge.net/ Data Set Repository) data from the CSU Stanislaus Library, the Intel

Research Lab in Seattle, the Edmonton Convention

Centre, and more.

IJRR (The International IJRR maintains a Web page for each article, often http://www.ijrr.org/contents/23\_12/  Journal of Robotics containing data and video of results. A good paper abstract/1113.html Research) example is by Bosse et al. [3], which has data

from Killian Court at MIT.

Conclusions

This article has described the SLAM problem and the essen tial methods for solving the SLAM problem and has summa rized key implementations and demonstrations of the method. While there are still many practical issues to over come, especially in more complex outdoor environments, the general SLAM method is now a well understood and established part of robotics. Part II of this tutorial will sum marize more recent work in addressing some of the remain ing issues in SLAM, including computation, feature representation, and data association.

References

[1] N. Ayache and O. Faugeras, “Building, registrating, and fusing noisy visual maps,” Int. J. Robot. Res., vol. 7, no. 6, pp. 45–65, 1988. [2] T. Bailey, J. Nieto, and E. Nebot, “Consistency of the FastSLAM algo rithm,” in Proc. IEEE Int. Conf. Robot. Automat., 2006.

[3] M. Bosse, P. Newman, J. Leonard, and S. Teller, “Simultaneous localiza tion and map building in large-scale cyclic environments using the Atlas framework,” Int. J. Robot. Res., vol. 23, no. 12, pp. 1113–1140, 2004.

[4] J.A. Castellanos, J.M. Martnez, J. Neira, and J.D. Tardós, “Experi ments in multisensor mobile robot localization and map building,” in Proc. 3rd IFAC Sym. Intell. Auton. Vehicles, 1998, pp. 173–178.

[5] J.A. Castellanos, J.D. Tardós, and G. Schmidt, “Building a global map of the environment of a mobile robot: The importance of correlations,” in Proc. IEEE Int. Conf. Robot. Automat., 1997, pp. 1053–1059.

[6] R. Chatila and J.P. Laumond, “Position referencing and consistent world modeling for mobile robots,” in Proc. IEEE Int. Conf. Robot. Automat., 1985, pp. 138–143.

[7] K.S. Chong and L. Kleeman, “Feature-based mapping in real, large scale environments using an ultrasonic array,” Int. J. Robot. Res., vol. 18, no. 1, pp. 3–19, 1999.

[8] D. Crisan and A. Doucet, “A survey of convergence results on particle filtering methods for practitioners,” IEEE Trans. Signal Processing, vol. 50, no. 3, pp. 736–746, 2002.

[9] J. Crowley, “World modeling and position estimation for a mobile robot using ultra-sonic ranging,” in Proc. IEEE Int. Conf. Robot. Automat., 1989, pp. 674–681.

[10] M. Csorba, “Simultaneous Localisation and Map Building,” Ph.D. dis sertation, Univ. Oxford, 1997.

[11] M. Csorba and H.F. Durrant-Whyte, “A new approach to simultaneous localisation and map building,” in Proc. SPIE Aerosense, Orlando, FL, 1996. [12] A.J. Davison, Y.G. Cid, and N. Kita, “Real-time 3D SLAM with wide-angle vision,” in Proc. IFAC/EURON Symp. Intell. Auton. Vehi cles, 2004.

[13] M. Deans and M. Hebert, “Experimental comparison of techniques for localization and mapping using a bearing-only sensor,” in Proc. Int. Symp. Experimental Robot., 2000, pp. 395–404.

JUNE 2006 IEEE Robotics & Automation Magazine 107 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.

[14] G. Dissanayake, P. Newman, H.F. Durrant-Whyte, S. Clark, and M. Csobra, “A solution to the simultaneous localisation and mapping (SLAM) problem,” IEEE Trans. Robot. Automat., vol. 17, no. 3, pp. 229–241, 2001.

[15] A. Doucet, “On sequential simulation-based methods for Bayesian fil tering,” Dept. Eng., Cambridge Univ., Tech. Rep.,1998. [16] H. Durrant-Whyte, D. Rye, and E. Nebot, “Localisation of automatic guided vehicles,” in Robotics Research: The 7th International Symposium (ISRR’95), G. Giralt and G. Hirzinger, Eds. New York: Springer Ver lag, pp. 613–625, 1996.

[17] H.F. Durrant-Whyte, “Uncertain geometry in robotics,” IEEE Trans. Robot. Automat., vol. 4, no. 1, pp. 23–31, 1988.

[18] R. Eustice, H. Singh, J. Leonard, M. Walter, and R. Ballard, “Visually navigating the RMS Titanic with SLAM information filters,” in Robot ics: Science and Systems. Cambridge, MA: MIT Press, 2005.

[19] J. Folkesson and H.I. Christensen, “Graphical SLAM—a self-correcting map,” in Proc. IEEE Int. Conf. Robot. Automat., 2004, pp. 791–798. [20] J. Guivant, E.M. Nebot, and S. Baiker, “Localization and map building

using laser range sensors in outdoor applications,” J. Robot. Syst., vol. 17, no. 10, pp. 565–583, 2000.

[21] J.E. Guivant and E.M. Nebot, “Optimization of the simultaneous localization and map-building algorithm for real-time implementa tion,” IEEE Trans. Robot. Automat., vol. 17, no. 3, pp. 242–257, 2001.

[22] J.S. Gutmann and K. Konolige, “Incremental mapping of large cyclic environments,” in Proc. IEEE Int. Symp. Comput. Intell. Robot. Automat., 1999, pp. 318–325.

[23] J. Hollerbach and D. Koditscheck, Eds., Robotics Research, The Ninth International Symposium (ISRR’99). New York: Springer-Verlag, 2000. [24] S.J. Julier and J.K. Uhlmann, “A counter example to the theory of simultaneous localization and map building,” in Proc. IEEE Int. Conf. Robot. Automat., pp. 4238–4243, 2001.

[25] J. Kim and S. Sukkarieh, “Airborne simultaneous localisation and map building,” in Proc. IEEE Int. Conf. Robot. Automat., 2003, pp. 406–411. [26] K. Konolige, “Large-scale map-making,” in Proc. Nat. Conf. AI (AAAI), 2004, pp. 457–463.

[27] J.J. Leonard and H.F. Durrant-Whyte, “Simultaneous map building and localisation for an autonomous mobile robot,” in Proc. IEEE Int. Work shop Intell. Robots Syst. (IROS), Osaka, Japan, 1991, pp. 1442–1447.

[28] J.J. Leonard and H.F. Durrant-Whyte, Directed Sonar Navigation. Norwell, MA: Kluwer, 1992.

[29] J.J. Leonard and H.J.S. Feder, “A computational efficient method for large-scale concurrent mapping and localisation,” in Robotics Research, The Ninth International Symposium (ISRR’99), J. Hollerbach and D. Koditscheck, Eds. New York: Springer-Verlag, pp. 169–176, 2000.

[30] J.J. Leonard, R.J. Rikoski, P.M. Newman, and M.C. Bosse, “Mapping partially observable features from multiple uncertain vantage points,” Int. J. Robot. Res., vol. 21, no. 10–11, pp. 943–975, 2002.

[31] P.S. Maybeck, Stochastic Models, Estimaton and Control, vol. I. New York: Academic 1979.

[32] M. Montemerlo, S. Thrun, D. Koller, and B. Wegbreit, “Fast-SLAM: A factored solution to the simultaneous localization and mapping prob lem,” in Proc. AAAI Nat. Conf. Artif. Intell., 2002, pp. 593–598.

[33] M. Montemerlo, S. Thrun, D. Koller, and B. Wegbreit, “Fast-SLAM 2.0: An improved particle filtering algorithm for simultaneous localiza tion and mapping that provably converges,” in Proc. Int. Joint Conf. Artif. Intell., 2003, pp. 1151–1156.

[34] K. Murphy, “Bayesian map learning in dynamic environments,” in Advances in Neural Information Processing Systems. San Mateo, CA: Mor gan Kaufman. 1999.

[35] J. Neira and J.D. Tardós, “Data association in stochastic mapping using the joint compatibility test,” IEEE Trans. Robot. Automat., vol. 17, no. 6, pp. 890–897, 2001.

[36] P.M. Newman and J.J. Leonard, “Pure range-only subsea SLAM,” in Proc. IEEE Int. Conf. Robot. Automat., 2003, pp. 1921–1926.

[37] P.M. Newman, J.J. Leonard, J. Neira, and J. Tardos, “Explore and return: Experimental validation of real time concurrent mapping and localiza tion,” in Proc. IEEE Int. Conf. Robot. Automat., 2002, pp. 1802–1809.

[38] W.D. Renken, “Concurrent localization and map building for mobile robots using ultrasonic sensors,” in Proc. IEEE Int. Workshop Intell. Robots Syst. (IROS), 1993.

[39] R. Smith and P. Cheesman, “On the representation of spatial uncer tainty,” Int. J. Robot. Res., vol. 5, no. 4, pp. 56–68, 1987.

[40] R. Smith, M. Self, and P. Cheeseman, “Estimating uncertain spatial relationships in robotics,” in Autonomous Robot Vehicles, I.J. Cox and G.T. Wilfon, Eds. New York: Springer-Verlag, pp. 167–193, 1990.

[41] S. Thrun, W. Bugard, and D. Fox, “A real-time algorithm for mobile robot mapping with applications to multi-robot and 3D mapping,” in Proc. Int. Conf. Robot. Automat., 2000, pp. 321–328.

[42] S. Thrun, D. Fox, and W. Burgard, “A probabilistic approach to con current mapping and localization for mobile robots,” Mach. Learning, vol. 31, no. 1, pp. 29–53, 1998.

[43] S. Thrun, Y. Liu, D. Koller, A. Ng, and H. Durrant-Whyte, “Simul taneous localisation and mapping with sparse extended information filters,” Int. J. Robot. Res., vol. 23, no. 7–8, pp. 693–716, 2004.

[44] C.C. Wang, C. Thorpe, and S. Thrun, “On-line simultaneous locali sation and mapping with detection and tracking of moving objects,” in Proc. IEEE Int. Conf. Robot. Automat., 2003, pp. 2918–2924.

[45] S.B. Williams, P. Newman, G. Dissanayake, and H.F. Durrant-Whyte, “Autonomous underwater simultaneous localisation and map build ing,” in Proc. IEEE Int. Conf. Robot. Automat. (ICRA), San Francisco, CA, Apr. 2000, pp. 1793–1798.

Hugh Durrant-Whyte received the B.Sc. in nuclear engineer ing from the University of London, United Kingdom, in 1983, and the M.S.E. and Ph.D. degrees, both in systems engineering, from the University of Pennsylvania, in 1985 and 1986, respec tively. From 1987–1995, he was a senior lecturer in engineering science, the University of Oxford, United Kingdom, and a fel low of Oriel College Oxford. From 1995–2002 he was a profes sor of mechatronic engineering at the University of Sydney. In 2002, he was awarded an Australian Research Council (ARC) Federation Fellowship. He also now leads the ARC Centre of Excellence in Autonomous Systems. His research work focuses on autonomous vehicle navigation and decentralized data fusion methods. His work in applications includes automation in cargo handling, mining, defense, and marine systems. He has pub lished over 300 technical papers and has won numerous awards and prizes for his work.

Tim Bailey received his B.E. in mechanical and mechatronic engineering at the University of Sydney in 1997 and his Ph.D. at the Australian Centre for Field Robots, University of Sydney, in 2003. He is currently a research fellow at the ACFR. His research interests include mobile robot localiza tion and mapping, Bayesian estimation techniques, and proba bilistic sensor modeling.

Address for Correspondence: Hugh Durrant-Whyte, Aus tralian Centre for Field Robotics (ACFR) J04, The Univer sity of Sydney, Sydney NSW 2006, Australia. E-mail: hugh@acfr.usyd.edu.au.

108 IEEE Robotics & Automation Magazine JUNE 2006 Authorized licensed use limited to: IEEE Xplore. Downloaded on September 23,2024 at 05:33:14 UTC from IEEE Xplore. Restrictions apply.